UN3系列铜铝管对焊机

UN3 Series Copper Tube and Aluminum Tube Butt Welding Machine

整机特点

UN3系列对焊机是为铜管与铝管连接专用的焊接设备，它可替代原先使用的专用高压闪光对焊机和钎焊。系统采用了可编程

序逻辑控制器（PLC）为动作逻辑控制单元，专用微电脑焊接控制器为焊接工艺控制单元，使用高品质铝合金气缸和高精度含油轴承与

精密机床用高精度光杆配合来保证机械动作的精度。具有可靠的焊接质量，控制与调整直观简便，生产效率高，无漏焊漏气现象，产成品率高等优点，同时克服了原来焊接工艺需要对焊接管内二次加工和工作电压高易出事故的缺点。

装有初级恒压和次级恒流的控制系统、强制水冷系统、气源储气罐，确保大量生产，焊接质量和焊接效果完全一致。

可储存多种不同规范，根据不同的管径调用不同的规范，操作简单，效率高，广泛应用于空调、冰箱、冷柜、冷库、压缩机等产品。

Main Features:

UN3 Series Copper Tube and Aluminum Tube Butt welding Machines specially designed for Copper Tube and Aluminum Tube welding, it can replace the High-voltage Flash Welding Machine and brazing machine with lower and old technology.

The System adopt Programmable Logic Controller(PLC) as the action logic controlling unit; adopt Special Microcomputer Welding Controller as the welding process control unit; adopt the High-quality Aluminum Alloy Cylinder and High-precision Oil Bearings integrated with High-precision Lathe Lever to make precise machinery actions.

It has the reliable welding quality, easy and simple control and adjustment, high productivity, no gas leakage phenomenon, and some other excellence.

The computerized controlling box with secondary constant voltage and constant current, force water cooling system and air storage tank can guarantee the machine produce mass weld in the same shape and quality.

Able to store many different Schedule Numbers according to different size of tubes,easy operation,high efficiency,Widely used in air conditioning ,refrigerator,freezer,refrigerator,compressor,etc.

主要技术参数 Main Technical Parameters

| 项目Item | 额定容量Rated Capacity | 额定输入电压Rated Input Voltage | 额定频率Rated Frequency | 最大短路电流 Max.Short Current | 负载持续率Rated Duty Cycle | 冷却水流量Cooling Water Rate | 平均 重量 Average Weight |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 型号Model |  |  |  |  |  |  |  |
| UN3-35 | 35KVA | 380V | 50/60Hz | 15000A | 50% | 150L/H | 350Kg |
| UN3-50 | 50KVA | 380V | 50/60Hz | 19000A | 50% | 150L/H | 410Kg |
| UN3-75 | 75KVA | 380V | 50/60Hz | 23500A | 50% | 200L/H | 480Kg |
| UN3-100 | 100KVA | 380V | 50/60Hz | 26000A | 50% | 200L/H | 530Kg |
| UN3-150 | 150KVA | 380V | 50/60Hz | 31000A | 50% | 300L/H | 620Kg |
| UN3-200 | 200KVA | 380V | 50/60Hz | 38000A | 50% | 300L/H | 730Kg |